- Dumpster to be of the square style or dockside style.  The Campus Physical Plant Trucking Department to be contacted as to the style and size.

- size: 8 cubic yard maximum

- sides: 12 gauge steel

- bottom: 10 gauge steel

- lids: (2) double wall plastic

- doors (2) 30” x 30” plastic (as applicable)

- side pockets: 7 gauge

- high bottom channels

- 2” drain holes with plugs

- bumper pads

- all inside welds continuous welded

- rust prohibitive prime coat inside, outside and underneath

- black finish of industrial alkyd enamel outside